THE CWLS RW CATALOG ON DISC

E. R. (Ross) Crain, P.Eng.

Spectrum 2000 Mindware Ltd

Calgary AB

First issued May 1987, Last revision Oct 1990. Published in Canadian Well Logging Society Journal, March 1991.

Introduction

For  a number of years, the Canadian Well Logging Society has published a catalog of  water resistivity values for Canada. Recently, this publication has been extensively  revised and the catalog has been printed as a series of 8½  x 11 inch maps covering Western Canada, Northwest Territories / Yukon, Arctic Islands, and East Coast Canada.

Large scale wall maps are also available from the Society.

A number of requests for the CWLS Rw Database in digital form have been received over the  last  few  years, and the Society has responded by publishing the database as an ASCII file for use  by log analysts in their own databases, and as a Lotus 1-2-3 file for use as a stand-alone database.

The CWLS expects that users of the ASCII database will write their  own search  and  retrieval  programs using a database management program of their own choice. The Lotus 1-2-3 program is provided only as a guide to the contents of the database and is not intended as a complete search and retrieval program.

Contents of the CWLS Rw Database

The files on these discs are a CWLS Shareware program and may be copied and distributed to others upon payment of a fee of $25.00 for each copy produced.   CWLS is not responsible for errors or omissions in the data or programs on these discs. The data and programs are protected by the copyright laws of Canada and the United  States, and  can only be reproduced under the terms and conditions described above.

The CWLS Rw Database contains 6000 records with the following fields:

MAP#  REC#     UNIQUE WELL ID    LATITUDE    LONGITUDE    RW@25'C

Each record represents one water resistivity value which has passed a series of criteria for  validity.  Those interested in the criteria should refer to the introductory text of the paper version of the CWLS Rw Catalog (1987).

The data discs contain the following files:

Disc #1: RWREADME.DOC which contains a copy of this paper

RWLIST.ASC which contains the ASCII file of the catalog

RWINDEX.WK1 which contains the Lotus 1-2-3 catalog index,

search macros, and print routines

Disc #2: RWCATL.WK1 which contains the same data as RWLIST.ASC, but in Lotus worksheet format.

If an ASCII file of the index is required, it can be created by printing the appropriate portions of RWCATL.WK1 to a disc file.

Using the Lotus 1-2-3 Rw Catalog

To  use the Lotus version of the CWLS Rw Catalog on a disc, create a directory called RWCAT on a disc drive of your choice. Copy RWCATL.WK1, RWINDEX.WK1,  and RWREADME.DOC to this directory. Load your LOTUS 1-2-3 program and change directory (/FD) to your RWCAT directory.  Then  load the RWINDEX.WK1 program with the Lotus File Retrieve (/FR) command. The following help screen will appear:

******************************************************************************

*										*

*		CANADIAN WELL LOGGING SOCIETY			*

*										*

*		FORMATION WATER RESISTIVITY CATALOG	*

*				HELP FILE					*

*										*

******************************************************************************

* INSTRUCTIONS								*

*    Press {Alt}S to search for and load individual map sets.		*

*    Press {Alt}P to print the loaded map set.				*

*    Press {Alt}I to print indexes.						*

*    Press {Alt}H to return to this screen.					*

*    Press /GV to view RW Map. Adjust scales to suit location desired.*

******************************************************************************

If you press {Alt}S the following screen appears:

******************************************************************************

*										*

*		CANADIAN WELL LOGGING SOCIETY			*

*										*

*		FORMATION WATER RESISTIVITY CATALOG	*

*				MAP INDEXES				*

*										*

*** Move Cursor Down To Desired Map Number and Press {Return} *

*										*

ALTA/BC		SASK/MAN		YUKON/NWT		*

ALBERTA		2  ALIDA		30  AKLAVIK		41

ARCS			15  AMARANTH	28  AKPAK		38

ARCTOMYS		20  ASHERN		34  ALMSTROM		41

BADHEART		2  ASHVILLE		22  ARCTIC RED		40

********************************************************************************

Move the cursor to the map number for the formation you want to view,

and press {Return}. The screen will clear while the file  loads  (about

12 seconds for a fast hard disk). A new screen appears:

******************************************************************************

*										*

*		CANADIAN WELL LOGGING SOCIETY			*

*										*

*	FORMATION WATER RESISTIVITY CATALOG		*

*				c. 1987 CWLS				*

*										*

******************************************************************************

*				MAP # XX					*

*										*

*	This section lists all the formation names included on the	*

*	map that was selected.						*

*										*

*										*

*********  Press {Page Down} To See Retrieved Data  *****************

If you press {Page Down}, the following screen appears:

*										*

* MAP#  REC#  UNIQUE WELL ID	LATITUDE	LONGITUDE	RW@25'C *

*											*

* 34	44	100091300126W100	49.03822	-100.82439	0.075	*

* 34	883	100133600419W100	49.34906	-99.91835	0.040	*

* 34	1236	100051300522W100	49.38638	-100.32505	0.045	*

* 34	1249	100072700524W100	49.41550	-100.63008	0.082	*

**************************************************************************************

You can scroll through the list with the arrow keys to find an RW near

your well. To view a map of the data, press /GV to view the RWMAP.  You

may need to use Lotus commands to change X and Y scales and ranges to

adjust map area.

DO NOT MESS WITH THE DATA OR MACROS UNLESS YOU KNOW LOTUS 1-2-3.

Press {Alt}I to print the index or {Alt}P to print the data from the

map area that was loaded with {Alt}S. Press {Alt}H to return to the

Help screen.  If there is no data in the catalog near your well, try

loading the maps adjacent to the one desired; ie. if you loaded Map #6,

try Map #5 and Map #7 also.

Using the ASCII Rw Catalog

You may load the ASCII file into any database manager program you wish,

such as dBase III or dBase IV, Oracle, Paradox, or any other  SQL  type

product.  Remember  that  CWLS  expects you to pay the Shareware fee of

$25.00 for each copy of the file that you put into a database. The CWLS

does not charge for each user of a non-commercial networked database,

which is a genuine bargain in today's software jungle.

To  use  the  data,  you  would have to write some database programs to

search and retrieve data from the database. The CWLS  encourages  users

to  share  these  programs with others, either as freeware, or as

shareware at a nominal fee, or as a technical article in the CWLS

Journal.

It also seems practical to load the Rw data file into a "Pop-up" or TSR

program on a DOS or OS/2 based PC, so that the  data  can  be  searched

while  retaining control of a running log analysis program. This should

be especially practical under Microsoft  Windows 3.0 or Presentation

Manager.  TSR  products  such as Lotus Metro or Sidekick may also work.

The CWLS invites contributions to the CWLS Journal describing any

successful attempts at this approach.

Third parties who sell the CWLS Rw Catalog in conjunction with their

database retrieval software must remit the CWLS Shareware fee or face

possible penalties.  Commercial database suppliers who wish to provide

the CWLS Rw Catalog over their database networks should contact the

CWLS for the appropriate licensing agreement.

If you receive a copy of the CWLS Rw Database from a friend or by

downloading a database, please be sure to send the $25.00 shareware fee

to the CWLS at 229, 640 5 Ave SW, Calgary AB T2P 3G4.

Conclusions

The CWLS Rw Catalog on Disc provides the basis for database retrieval

of high quality Canadian water resistivity values for the first time

for many people at a very low price. The Lotus 1-2-3 program provides a

quicklook retrieval program and the ASCII file provides access from

more sophisticated DBMS programs. Third party programs are expected to

enhance the utility of the data.

c. 1987 - 1990 CWLS